SECTION 08 88 53

TSS SchoolDefender® Security Glass

(Specifier Note:  The purpose of this guide specification is to assist the Specifier in correctly specifying security glazing with its installation in security windows.

The Specifier must edit this guide specification to fit the needs of each specific project.  References have been made within the text of the specification to MasterFormat section numbers and titles. The Specifier must coordinate these numbers and titles with sections included for the specific project.

Throughout the guide specification, there are Specifier Notes to assist in the editing of the file.  Brackets have been used to indicate when a selection is required. Contact a TSS representative for further assistance with appropriate product selections.)

PART 1 - GENERAL

  - SECTION INCLUDES

##### Forced-entry Laminated Security Glass.

1.2  REFERENCES

##### ASTM F1233-08 Standard Test for Security Glazing Materials.

##### ASTM C1036: Standard Specifications for Flat Glass

##### ASTM C1349: Standard Specification for Architectural Flat Glass Clad Polycarbonate

ANSI Z97.1: Safety Materials Used in Buildings

1.3 ACTION SUBMITTALS

- Refer to Section [01 33 00 Submittal Procedures] [Insert section number and title].

Product Data: Including manufacturer recommended installation instructions.

Shop Drawings:  Include plans, elevations, sections, details, attachment to other work.

Samples:  For each exposed glazing type.

  - INFORMATION SUBMITTALS

- Test Reports: Submit complete, independent laboratory test reports for the exact product being proposed. The report must confirm the product meets or exceeds ASTM F1233 Class 2.0 Body Passage. Summary sheets or marketing brochures are not acceptable proof of performance.

- Warranty:  Sample of warranty

1.5 CLOSEOUT SUBMITTALS

        - Refer to Section [01 78 00 Closeout Submittals] [Insert section number and title].

        - Maintenance data.

1.6 WARRANTY

        - Workmanship Warranty: All materials shall be warranted against defects for a period of [1] year for the date of receipt at the project site.  Provide certificates of manufacturer’s standard limited warranty with closeout documents.

        - Finish Warranty: Manufacturer’s warranty against deterioration of factory finishes for the period of [1] year from the date of Substantial Completion.

(Specifier Note: Product information is proprietary to TSS. If additional products are required for competitive procurement, contact TSS for assistance in listing competitive products that may be available.)

PART 2 - PRODUCTS

2.1 	MANUFACTURED UNITS

##### Basis of Design Product:

###### SchoolDefender® C2.0 FE by Total Security Solutions. Substitutions: Permitted only if the proposed product provides documented evidence of meeting or exceeding the specified performance requirements in Section 1.4.

###### Subject to compliance with requirements, provide products by the following:

- Total Security Solutions, Inc., 935 Garden Lane, Fowlerville, MI 48836, 866 734-6277. Attn: Sales Department, contact@tssbulletproof.com. Web: www.tssbulletproof.com.

  - PERFORMANCE REQUIREMENTS

- Glazing Construction: Laminated glass-clad polycarbonate configuration consisting of multiple lites of glass and an inner core of polycarbonate and proprietary security interlayers.

#### 2.4 SCHOOLDEFENDER C2.0 FE GLAZING

##### Glazing shall be as shown on the drawings or as specified separately in [08 88 53 Security Glazing] [Insert section number and title].

##### SchoolDefender standard makeups.

###### SchoolDefender® 7/16” C2.0 FE  |  PART#: DEF-050-C20FE-T-MFG

###### SchoolDefender® 1.00” IG C2.0 FE  |  PART#: DEF-IG100-C20FE-T-MFG

###### SchoolDefender® 1.125” IG C2.0 FE  |  PART#: DEF-IG113-C20FE-T-MFG

#### PART 3 - EXECUTION

##### DELIVERY, STORAGE, AND HANDLING

##### Deliver, store, and handle materials under provisions of Section 01 60 00 [Insert section number and title].

##### Deliver materials to specified destination in manufacturer or distributor’s packaging, undamaged, complete with installation instructions.

##### SchoolDefender must not be exposed outside the range -40 degrees F to 120 degrees F (-40 degree C to +50 degrees C) during storage and transportation.

##### Store off ground, under cover, protected from weather and construction activities.

##### Store sheets of glass vertically. DO NOT lean.

##### PREPARATION

##### Verify that existing frames and glazing pockets are prepared and capable of accommodating the required glazing thickness and weight without structural modification to the frame.

##### Prior to beginning installation, verify that areas have been prepared as required by the Contract Documents and architectural drawings, and Shop Drawings have been approved.

##### Notify Architect of any unsatisfactory preparation that is responsibility of others.

##### Clean and prepare all surfaces per manufacturers’ recommendations as required for achieving the best results for the substrate under the project conditions.

##### Do not begin installation of material until all unsatisfactory conditions have been resolved and approved by Architect.

#### 3.2 INSTALLATION

##### Do not begin installation until areas have been verified and surfaces properly prepared in accordance with Drawings.

##### Installation in accordance with manufacturer’s instructions and ASTM F1233. Set all equipment plumb.

##### Apply sealant in accordance with manufacturer's recommendations as indicated in installation instructions.

##### Remove excess sealant and leave exposed surfaces clean and smooth

#### 3.3 PROTECTION & CLEANING

##### Protect material from damage during ongoing construction operations. If damage occurs, remove and replace as required.

##### Inspection: Verify installation is complete and complies with manufacturer’s requirements.

##### Provide final cleaning of products and accessories, removing excess sealant, labels and protective covers.

##### Cleaning: Clean polycarbonate and glass surfaces using only manufacturer-approved mild soap and water solutions. Do not use ammonia-based, abrasive, or harsh chemical cleaners, which can damage the polycarbonate substrate.

##### Touch-up, repair or replace damaged products prior to Substantial Completion.

END OF SECTION